Renaissance Festival Tunic





Measurements





Direct Measurements required (from person)





Number�
Name�
Measurement�
�
1�
Wrist to Wrist�
�
�
2�
Shoulder Width�
�
�
3�
Desired Length�
�
�
4�
Upper Arm Circumference�
�
�
5�
Chest�
�
�
6�
Waist�
�
�
7�
Desired Neckline Circumference�
�
�
8�
Desired Wrist Circumference�
�
�
9�
Desired Neck Opening Length�
�
�



Calculated Measurements required





Number�
Name�
Calculation�
Measurement�
�
10�
Neck Radius�
(#7)/6.28�
�
�
11�
Armhole Length�
((#4) + 10)/2�
�
�
12�
Armhole Depth�
((#17) - (#2))/2�
�
�
14�
Sleeve Length�
(#1)/2 - (#2)/2�
�
�
15�
Wrist Half-Width�
(#8)/2�
�
�
16�
Underarm Tunic Length�
(#3) - (#11)�
�
�
17�
Tunic Width�
((#5) + 12)/2�
�
�



Materials





Solid Color Fabric 45" wide, length ((2 x (#3)) + 2) + 2 x ((#11) x 2) + 6"


Matching Thread


Matching Bias Tape length (#7) + 6"


Contrasting Leather Lace


 �
Steps 





��������������������������������Cut length of fabric that is (2 x (#3)) +2" long and (#17) + 1" wide for the body of the tunic.  Also cut out insets for armholes and neck, outlined in red.  Dashed lines indicate measurement boundaries and are for reference only.












































�����������������Cut two lengths of fabric that are (#14) long and (#11) x 2 wide for sleeves.






































Cut two rectangles of fabric (#9) + 2" long by 6" wide for the backing to the neck opening.





Attach sleeves to tunic body at armholes: Use 0.5" seam on short angled section of sleeves against angled portion of tunic.  Repeat for other side.





Seam tunic sides and sleeves together:  Make one seam from bottom of tunic length, up to sleeve, and across underside of sleeve.  Repeat for other side.





Hem sleeves (1/4", 3/8").





Hem tunic bottom (1/4", 3/8").





Neck opening backing: Seam the two (#9) + 2" x 6" rectangles at 3/8" on three sides (two (#9) + 2, one 6").  Turn inside out and press flat.





Attach neck opening backing to front of tunic:  Center neck opening backing on front of tunic (whichever side of tuni
